Dandy Vision Scanner – Copy Denture Diagnostic Workflow

# 1. Problem Statement

This workflow addresses a current technical limitation of the Dandy Vision Scanner.

- Dandy Vision does not generate scan streams (video) within the raw scan file.

- Without this scan stream, Chairside Copy & Paste of scans into a new order is not supported.

- As a result, prior 360° scans cannot be loaded directly into the Copy/Reference workflow from an existing diagnostic scan.

- This functionality is supported on Trios 3 scanners, but not yet on Vision.

- EPD is actively working on enabling this capability for Vision.

# 2. Why This Matters

For copy denture cases:

- Practices need to reuse existing denture scans (360°).

- The inability to reference prior scans directly adds friction and confusion.

- A clear workaround is required to ensure cases can still be ordered successfully with Vision.

# 3. Current-State Solution (Recommended Workaround)

The most reliable and reasonable option with the current software state is:

Diagnostic Scan → Download → Import Workflow (Copy Only)

This approach allows practices to complete copy denture cases while remaining within Vision's technical constraints.

# 4. Workflow Overview

| Step 1: Diagnostic Scan | → | Step 2: Download (.3oxz) | → | Step 3: Upload & Submit |
| --- | --- | --- | --- | --- |

# 5. End-to-End Workflow

## A. Diagnostic Scanning (Vision Scanner)

- Start a New Diagnostic Scan

- Select Dual Arch setup

- Scan the existing denture(s) extraorally (360°)

- Capture intraoral bite scans as needed

- Complete and submit the Diagnostic Order

- Wait for the scan to finish processing

## B. Download Diagnostic Scans

- Open a Web Browser (not the Vision scanning software) — Any computer with internet access is acceptable.

- Navigate to the Dandy Portal.

- Go to the Patients Tab

- Locate the patient's Diagnostic Order.

- Download the Diagnostic Scans

- File format: .3oxz

## C. Create the Copy Denture Order

- Navigate to the Orders Tab in the Dandy Portal.

- Click New Order.

- Select Copy Denture as the order type.

- Complete the Rx details as required.

- When prompted, Upload the .3oxz File — This file contains the previously captured diagnostic scans.

- Submit the Order.

# 6. What's Needed for Scale

## Enablement & Training

Additional Enablement from Training resources on:

- Denture-specific diagnostic scanning workflows

- Copy denture use cases with Vision

# 7. Key Notes & Callouts

- This workflow is a temporary but supported workaround.

- Must use a web browser for downloading and uploading scans.

- Chairside copy/paste is not currently supported on Vision.

- Future EPD enhancements may simplify or eliminate this process.

# 8. Quick Reference Summary

| Phase | Action | Tool/Location |
| --- | --- | --- |
| Diagnostic | Scan & submit diagnostic order | Vision Scanner |
| Download | Download diagnostic scans | Dandy Portal (Patients Tab) |
| Order | Create copy denture order | Dandy Portal (Orders Tab) |
| Upload | Upload .3oxz file & submit | Dandy Portal (Copy Order) |